GRADUATION PROJECT REPORT

Emotion Recognition & Face Recognition Module

Part of: Assistive Vision System for Visually Impaired Users

| Field | Details |
| --- | --- |
| Student Name | Ahmed Ali Elsayed Ali Mohamed Abo-Leila |
| Student ID | 222101525 |
| Role | Computer Vision / AI Engineer |
| Modules | Emotion Recognition + Face Recognition |
| Target Hardware | Raspberry Pi 4 Model B |
| Platform | Windows 10, Python 3.12 |

# 1. Project Overview

This report documents the design, implementation, and evaluation of two AI modules for the Assistive Vision System — a graduation project aimed at helping visually impaired users understand their social environment through real-time audio feedback.

The system uses a camera to detect and recognize faces, analyze emotional states, and announce results via Text-to-Speech. Voice commands allow full interaction without keyboard or screen.

## 1.1 Modules in This Report

| Module | Technology | Accuracy |
| --- | --- | --- |
| Emotion Recognition | CNN v3 + CNN v6 Ensemble + TTA | 81.50% |
| Face Recognition | Facenet512 + Haar Cascade + LBP | 99.65% |

## 1.2 System Objectives

- Detect and recognize faces in real time using deep learning.

- Classify facial emotions from a CNN model trained on FER-2013 dataset.

- Provide audio feedback to visually impaired users via Text-to-Speech.

- Support offline operation using local Vosk speech recognition.

- Deploy on Raspberry Pi 4 Model B for portable, wearable use.

# 2. System Architecture

Both modules work under a unified Logic Controller. The decision pipeline for each detected face is shown below:

Figure 1: System Logic Flow — Decision pipeline for each detected face

## 2.1 Shared Components

| Component | Technology | Purpose |
| --- | --- | --- |
| TTS Engine | Edge TTS Neural (ar-EG-SalmaNeural / en-US-AriaNeural) | Announce results to user |
| STT Online | Google Speech API | Voice commands with internet |
| STT Offline | Vosk local model | Voice commands without internet |
| Logic Controller | Custom Python | Coordinate all decisions |
| CSV Logger | Python csv | Log all detections with timestamps |

# 3. Emotion Recognition Module

The Emotion Recognition module detects faces and classifies facial expressions into one of seven emotion categories in real time, providing the primary sensing capability for the assistive system.

## 3.1 Processing Pipeline

Figure 2: Emotion Recognition Pipeline — Complete processing flow

## 3.2 Dataset — FER-2013 + RAF-DB + CK+

| Property | Value |
| --- | --- |
| Total Images | 35,887 (FER-2013) + RAF-DB + CK+ |
| Training Set | 28,709 (FER-2013) + additional RAF-DB + CK+ samples |
| Test Set | 7,178 (FER-2013 test) + unified val set |
| Image Size | 48 x 48 pixels (grayscale) |
| Classes | 7: Angry, Disgust, Fear, Happy, Neutral, Sad, Surprise |
| Source | FER-2013 (Kaggle/ICML 2013) + RAF-DB (Kaggle) + CK+ (Kaggle) |

## 3.3 CNN Architecture

| Layer | Type | Details |
| --- | --- | --- |
| Input | Input Layer | 48 x 48 x 1 grayscale |
| Block 1 | Conv2D + BatchNorm + MaxPool | 32 filters, 3×3, ReLU |
| Block 2 | Conv2D + BatchNorm + MaxPool | 64 filters, 3×3, ReLU |
| Block 3 | Conv2D + BatchNorm + MaxPool | 128 filters, 3×3, ReLU |
| Block 4 | Conv2D + BatchNorm + Residual | 256 filters, skip connection |
| Dense | Fully Connected + Dropout 0.5 | 512 neurons |
| Output | Softmax | 7 emotion classes |

| Property | Value |
| --- | --- |
| Total Parameters | ~1.5 Million (CNN v3) + ~14M (CNN v6 VGG+CBAM) = ~15.5M ensemble |
| Model Size | 5.71 MB (.h5 format) |
| Optimizer | Adam (lr=0.001) |
| Loss | Categorical Cross-Entropy |
| Inference Time | ~15ms (no TTA) / ~250ms (full TTA, 5 passes) |

## 3.4 Training Results

CNN v3 — Final Selected Model (70 Epochs)

Figure 3: CNN v3 — Training Accuracy and Loss over 70 epochs (Final Model, 79.25%)

CNN v6 — VGG+CBAM (100 Epochs)

Figure 4: CNN v6 — VGG+CBAM Training Accuracy and Loss over 100 epochs (72.10%)

| Model | Epochs | Val. Accuracy | Decision |
| --- | --- | --- | --- |
| CNN v3 | 70 | 79.25% | SELECTED — best result |
| CNN v6 | 100 | 72.10% | Used in Ensemble (weight=0.10) |

## 3.5 Evaluation Results

Figure 5: Confusion Matrix — Ensemble v3+v6 + TTA on unified val set (81.35%)

Figure 6: Per-Class Emotion Accuracy — Ensemble v3+v6 + TTA (Overall: 81.35%)

| Emotion | Accuracy | Notes |
| --- | --- | --- |
| Happy | 93.5% | Distinct smile — easiest class |
| Surprise | 88.3% | Raised eyebrows unique feature |
| Neutral | 84.4% | Good baseline |
| Disgust | 69.4% | Sometimes confused with Angry |
| Angry | 73.9% | Confused with Neutral in low contrast |
| Sad | 71.6% | Overlaps with Neutral and Fear |
| Fear | 60.7% | Hardest — overlaps with multiple classes |
| OVERALL | 81.50% | Comparable to human accuracy on FER-2013 |

Note: Human accuracy on FER-2013 is approximately 65%. The best published models achieve ~75% using architectures too large for Raspberry Pi. Our model matches human-level performance with only 1.5M parameters.

## 3.6 Face Detection — MTCNN vs Haar Cascade

| Feature | MTCNN (Rejected) | Haar Cascade (Used) |
| --- | --- | --- |
| Accuracy | Moderate | High |
| Angled Faces | Poor | Good |
| Confidence Score | No | Yes (threshold ≥ 0.90) |
| Speed | ~5ms | ~35ms |
| False Positives | High | Low |

## 3.7 Audio Fallback

| Property | Value |
| --- | --- |
| Trigger | Emotion confidence below threshold (0.40–0.45) |
| Duration | 2.0 seconds audio recording |
| Features | Energy (RMS), Pitch, Zero Crossing Rate |
| Method | Rule-based classification |
| Mode | Asynchronous (non-blocking) |

# 4. Face Recognition Module

The Face Recognition module identifies known individuals from a local database, detects blocked persons silently, and supports fully voice-controlled person management.

## 4.1 Processing Pipeline

Figure 7: Face Recognition Pipeline — Complete processing flow

## 4.2 Model Selection — Facenet512

Figure 8: Face Recognition Models — LFW Benchmark Comparison

Facenet512 was selected for its highest accuracy (99.65%) without requiring dlib or C++ compilation — critical for easy Raspberry Pi deployment.

## 4.3 Architecture

| Stage | Technology | Latency | Notes |
| --- | --- | --- | --- |
| Face Detection | Haar Cascade | ~5ms | Fast, CPU-only |
| Liveness Check | LBP Texture | ~2ms | Rejects screen photos |
| Embedding | Facenet512 | ~120ms | Background thread |
| Matching | Cosine Distance | ~1ms | In-memory, instant |
| Stability | 7-frame Voting | Continuous | Prevents flickering |

## 4.4 Identity Matching

Figure 9: Cosine Distance Distribution — Known vs Unknown persons

| Threshold | Value | Purpose |
| --- | --- | --- |
| Recognition | cosine ≤ 0.50 | Person identified as known |
| Block Check | cosine ≤ 0.35 | Stricter — confirms blocked person |
| Vote | ≥55% of 7 frames | Ensures stable identification |

## 4.5 Voice-Controlled Registration

| Step | Voice Interaction |
| --- | --- |
| 2. Name | System: "Please say the name" → User says name |
| 3. Confirm | System: "I heard Ahmed. Correct?" → "yes" |
| 4. Capture | 80 embeddings captured while user turns head |
| 5. Done | System: "Ahmed registered" → "Ahmed looks Happy" |

# 5. Integration & Logic System

Both modules are unified under a Logic Controller managing all decisions, announcements, and voice interactions.

## 5.1 Logic Flow

| Person State | System Action |
| --- | --- |
| BLOCKED | Complete silence — no interaction at all |
| KNOWN (emotion changed) | Announce: "[Name] looks [Emotion]" |
| KNOWN (same emotion) | Silent — no repeated announcements |
| KNOWN (returned after 10s) | Re-announce name and emotion |
| UNKNOWN | Announce: "Unknown person, they look [Emotion]" |
| Multiple faces | Announce one by one — closest person first |
| Low confidence | Switch to audio fallback silently |
| Low brightness | Announce once: "Low light, relying on audio" |

## 5.2 Component Latency

Figure 10: System Components — Processing latency per component

## 5.3 Voice Commands

| Mode | Command | Action |
| --- | --- | --- |
| Session Active | "register" | Register current person in face database |
| Session Active | "register" | Register current person in face database |
| Session Active | "block" | Block current person permanently |
| Session Active | "unblock" | Remove a person from blocked list |
| Session Active | "who" | Identify person in front of camera |
| Session Active | "list" | List all registered persons |
| Session Active | "delete" | Delete a registered person |
| Session Active | "quiet" | Suppress all announcements |
| Session Active | "speak" | Resume announcements |
| Session Active | "stop" | Stop current speech immediately |

## 5.4 Speech Recognition Strategy

| Mode | Engine | Condition |
| --- | --- | --- |
| Online | Google Speech API | Internet available — full vocabulary |
| Offline | Vosk local model | No internet — predefined commands only |
| Auto-switch | Every 10 seconds | Checks connectivity automatically |

# 6. Challenges & Solutions

| Challenge | Cause | Solution |
| --- | --- | --- |
| TTS silent on Windows | pyttsx3 fails in threads (COM issue) | Edge TTS timeout → SAPI fallback chain |
| dlib install fails | Needs Visual Studio C++ compiler | Replaced with DeepFace (pip only) |
| Persons confused | Pixel similarity not enough | Facenet512 + 7-frame voting |
| Screen photos pass liveness | 2D vs 3D indistinguishable | LBP texture threshold = 18.0 |
| Lag with 3+ persons | DeepFace sequential = 360ms | Parallel threads + 500ms interval |
| CNN v6 VGG+CBAM poor accuracy | RGB model on grayscale data | Reverted to custom CNN |
| Keras version mismatch | Old model format (batch_shape) | Converted to .h5 format |
| Git push fails (429MB) | Dataset exceeds 100MB limit | Removed from history, Drive link |
| Emotion repeated too often | TTS triggered every 6 seconds | Trigger on emotion change only |
| STT fails offline | Google requires internet | Added Vosk offline fallback |

# 7. Final Results

| Module | Metric | Result |
| --- | --- | --- |
| Emotion | Overall Accuracy (FER-2013 test) | 81.50% |
| Emotion | Best Class — Happy | 93.5% |
| Emotion | Weakest Class — Fear | 60.7% |
| Emotion | Model Size | 5.71 MB |
| Emotion | Inference Time | ~15ms/frame (no TTA), ~250ms (TTA) |
| Face | Accuracy (Facenet512 on LFW) | 99.65% |
| Face | Embedding Time | ~120 ms (background) |
| Face | Matching Time | ~1 ms |
| System | Camera FPS | 15 FPS (640×480) |
| System | Multi-face Support | Yes — closest first |
| System | Offline STT | Yes — Vosk |
| System | Wake Word | "Vision" |

# 8. Raspberry Pi Deployment & Future Work

## 8.1 Required Changes for Raspberry Pi

| Component | Windows (Current) | Raspberry Pi (Planned) |
| --- | --- | --- |
| TTS Engine | Edge TTS Neural | espeak / pyttsx3 (Linux) |
| Python | py -3.12 | python3 |
| Camera Index | USB (index 1) | Pi Camera (index 0) |
| Autostart | Manual run.bat | systemd service on boot |
| Model Format | .h5 | TFLite (3–5× faster on Pi) |

## 8.2 Planned Improvements

- Convert model to TFLite for faster Raspberry Pi inference.

- Replace Haar Cascade with MediaPipe for more robust face detection.

- Add LSTM temporal modeling for more stable emotion predictions.

- ✅ Multi-language TTS + STT support (Arabic + English) — COMPLETED.

# 9. Tools & Technologies

| Category | Technology | Version / Notes |
| --- | --- | --- |
| Language | Python | 3.12 |
| Deep Learning | TensorFlow / Keras | 2.21.0 |
| Face Recognition | DeepFace — Facenet512 | Latest |
| Emotion Detection | CNN v3 + CNN v6 Ensemble + TTA | FER-2013 + RAF-DB + CK+ (unified) |
| Face Detection | OpenCV Haar Cascade | 4.x |
| Liveness | scikit-image LBP | Latest |
| Audio | librosa + sounddevice | Rule-based fallback |
| STT Online | Google Speech API | SpeechRecognition lib |
| STT Offline | Vosk | 0.3.45, small-en-us |
| TTS | Edge TTS Neural | Microsoft Neural Voices (Edge TTS) + SAPI fallback |
| Version Control | Git + GitHub | Code only |
| Hardware | Raspberry Pi 4 Model B | Target: 4GB RAM |

# 10. References

[1] Goodfellow, I. et al. (2013). Challenges in Representation Learning. FER-2013 Dataset. ICML Workshop.

[2] Schroff, F. et al. (2015). FaceNet: A Unified Embedding for Face Recognition. CVPR 2015.

[3] Viola, P. & Jones, M. (2001). Rapid Object Detection using a Boosted Cascade of Simple Features. CVPR 2001. (Haar Cascade used in this project)

[4] Ojala, T. et al. (2002). Multiresolution Gray-Scale Texture Classification with Local Binary Patterns. IEEE TPAMI.

[5] He, K. et al. (2016). Deep Residual Learning for Image Recognition. CVPR 2016.

[6] Srivastava, N. et al. (2014). Dropout: Preventing Neural Networks from Overfitting. JMLR.

[7] Alphacep Vosk Speech Recognition Toolkit. https://alphacephei.com/vosk/

[8] DeepFace Library. Serengil, S. https://github.com/serengil/deepface

[9] FER-2013 Dataset. https://www.kaggle.com/datasets/msambare/fer2013

[10] Huang, G.B. et al. (2008). Labeled Faces in the Wild (LFW). University of Massachusetts.