GRADUATION PROJECT REPORT

Emotion Recognition & Face Recognition Module

Part of: Assistive Vision System for Visually Impaired Users

| Field | Details |
| --- | --- |
| Student Name | Ahmed Ali Elsayed Ali Mohamed Abo-Leila |
| Student ID | 222101525 |
| Role | Computer Vision / AI Engineer |
| Modules | Emotion Recognition + Face Recognition |
| Target Hardware | Raspberry Pi 4 Model B |
| Platform | Windows 10, Python 3.12 |

# 1. Project Overview

This report documents the design, implementation, and evaluation of two AI modules for the Assistive Vision System — a graduation project aimed at helping visually impaired users understand their social environment through real-time audio feedback.

The system uses a camera to detect and recognize faces, analyze emotional states, and announce results via Text-to-Speech. Voice commands allow full interaction without keyboard or screen.

## 1.1 Modules in This Report

| Module | Technology | Accuracy |
| --- | --- | --- |
| Emotion Recognition | CNN v3 + CNN v6 Ensemble + TTA | 81.35% |
| Face Recognition | Facenet512 + Haar Cascade + LBP | 99.63% |

## 1.2 System Objectives

- Detect and recognize faces in real time using deep learning.

- Classify facial emotions from a CNN model trained on FER-2013 dataset.

- Provide audio feedback to visually impaired users via Text-to-Speech.

- Support offline operation using local Vosk speech recognition.

- Deploy on Raspberry Pi 4 Model B for portable, wearable use.

# 2. System Architecture

Both modules work under a unified Logic Controller. The decision pipeline for each detected face is shown below:

Figure 1: System Logic Flow — Decision pipeline for each detected face

## 2.1 Shared Components

| Component | Technology | Purpose |
| --- | --- | --- |
| TTS Engine | Edge TTS Neural — Online (ar-EG-SalmaNeural / en-US-AriaNeural) | Microsoft Neural Voices (Online) + SAPI/espeak Offline fallback |
| STT Online | Google Speech API | Voice commands with internet |
| STT Offline | Vosk local model | Voice commands without internet |
| Logic Controller | Custom Python | Coordinate all decisions |
| CSV Logger | Python csv | Log all detections with timestamps |

# 3. Emotion Recognition Module

The Emotion Recognition module detects faces and classifies facial expressions into one of seven emotion categories in real time, providing the primary sensing capability for the assistive system.

## 3.1 Processing Pipeline

Figure 2: Emotion Recognition Pipeline — Complete processing flow

## 3.2 Dataset — FER-2013 + RAF-DB + CK+

| Property | Value |
| --- | --- |
| Total Images | 35,887 (FER-2013) + RAF-DB + CK+ |
| Training Set | 28,709 (FER-2013) + additional RAF-DB + CK+ samples |
| Test Set | 7,178 (FER-2013 test) + unified val set |
| Image Size | 48 x 48 pixels (grayscale) |
| Classes | 7: Angry, Disgust, Fear, Happy, Neutral, Sad, Surprise |
| Source | FER-2013 (Kaggle/ICML 2013) + RAF-DB (Kaggle) + CK+ (Kaggle) |

## 3.3 CNN Architecture

| Layer | Type | Details |
| --- | --- | --- |
| Input | Input Layer | 48 x 48 x 1 grayscale |
| Block 1 | Conv2D + BatchNorm + MaxPool | 32 filters, 3×3, ReLU |
| Block 2 | Conv2D + BatchNorm + MaxPool | 64 filters, 3×3, ReLU |
| Block 3 | Conv2D + BatchNorm + MaxPool | 128 filters, 3×3, ReLU |
| Block 4 | Conv2D + BatchNorm + Residual | 256 filters, skip connection |
| Dense | Fully Connected + Dropout 0.5 | 512 neurons |
| Output | Softmax | 7 emotion classes |

| Property | Value |
| --- | --- |
| Total Parameters | ~2.5 Million (CNN v3) + ~14M (CNN v6 VGG+CBAM) = ~15.5M ensemble |
| Model Size | ~18.3 MB (.h5 format) |
| Optimizer | Adam (lr=0.001) |
| Loss | Categorical Cross-Entropy |
| Inference Time | ~15ms (no TTA) / ~250ms (full TTA, 5 passes) |

## 3.4 Training Results

CNN v3 — Final Selected Model (70 Epochs)

Figure 3: CNN v3 — Training Accuracy and Loss over 70 epochs (Final Model, 79.25%)

CNN v6 — VGG+CBAM (100 Epochs)

Figure 4: CNN v6 — VGG+CBAM Training Accuracy and Loss over 100 epochs (72.12%)

| Model | Epochs | Val. Accuracy | Decision |
| --- | --- | --- | --- |
| CNN v3 | 70 | 79.25% | SELECTED — best result |
| CNN v6 | 100 | 72.12% | Used in Ensemble (weight=0.10) |

## 3.5 Evaluation Results

Figure 5: Confusion Matrix — Ensemble v3+v6 + TTA on unified val set (81.35%)

Figure 6: Per-Class Emotion Accuracy — Ensemble v3+v6 + TTA (Overall: 81.35%)

| Emotion | Accuracy | Notes |
| --- | --- | --- |
| Happy | 93.5% | Distinct smile — easiest class |
| Surprise | 88.3% | Raised eyebrows unique feature |
| Neutral | 84.4% | Good baseline |
| Disgust | 69.4% | Sometimes confused with Angry |
| Angry | 73.9% | Confused with Neutral in low contrast |
| Sad | 71.6% | Overlaps with Neutral and Fear |
| Fear | 60.7% | Hardest — overlaps with multiple classes |
| OVERALL | 81.35% | Comparable to human accuracy on FER-2013 |

Note: Human accuracy on FER-2013 is approximately 65%. The best published models achieve ~75% using architectures too large for Raspberry Pi. Our model matches human-level performance with only 2.5M parameters.

## 3.6 Face Detection — MTCNN vs Haar Cascade

| Feature | MTCNN (Rejected) | Haar Cascade (Used) |
| --- | --- | --- |
| Accuracy | Moderate | High |
| Angled Faces | Poor | Good |
| Confidence Score | No | Yes (threshold ≥ 0.90) |
| Speed | ~35ms | ~5ms |
| False Positives | High | Low |

## 3.7 Audio Fallback

| Property | Value |
| --- | --- |
| Trigger | Emotion confidence below threshold (0.40–0.45) |
| Duration | 2.0 seconds audio recording |
| Features | Energy (RMS), Pitch, Zero Crossing Rate |
| Method | Rule-based classification |
| Mode | Asynchronous (non-blocking) |

# 4. Face Recognition Module

The Face Recognition module identifies known individuals from a local database, detects blocked persons silently, and supports fully voice-controlled person management.

## 4.1 Processing Pipeline

Figure 7: Face Recognition Pipeline — Complete processing flow

## 4.2 Model Selection — Facenet512

Figure 8: Face Recognition Models — LFW Benchmark Comparison

Facenet512 was selected for its highest accuracy (99.63%) without requiring dlib or C++ compilation — critical for easy Raspberry Pi deployment.

## 4.3 Architecture

| Stage | Technology | Latency | Notes |
| --- | --- | --- | --- |
| Face Detection | Haar Cascade | ~5ms | Fast, CPU-only |
| Liveness Check | LBP Texture | ~2ms | Rejects screen photos |
| Embedding | Facenet512 | ~120ms | Background thread |
| Matching | Cosine Distance | ~1ms | In-memory, instant |
| Stability | 7-frame Voting | Continuous | Prevents flickering |

## 4.4 Identity Matching

Figure 9: Cosine Distance Distribution — Known vs Unknown persons

| Threshold | Value | Purpose |
| --- | --- | --- |
| Recognition | cosine ≤ 0.50 | Person identified as known |
| Block Check | cosine ≤ 0.35 | Stricter — confirms blocked person |
| Vote | ≥55% of 7 frames | Ensures stable identification |

## 4.5 Voice-Controlled Registration

| Step | Voice Interaction |
| --- | --- |
| 2. Name | System: "Please say the name" → User says name |
| 3. Confirm | System: "I heard Ahmed. Correct?" → "yes" |
| 4. Capture | 80 embeddings captured while user turns head |
| 5. Done | System: "Ahmed registered" → "Ahmed looks Happy" |

# 5. Integration & Logic System

Both modules are unified under a Logic Controller managing all decisions, announcements, and voice interactions.

## 5.1 Logic Flow

| Person State | System Action |
| --- | --- |
| BLOCKED | Complete silence — no interaction at all |
| KNOWN (emotion changed) | Announce: "[Name] looks [Emotion]" |
| KNOWN (same emotion) | Silent — no repeated announcements |
| KNOWN (returned after 10s) | Re-announce name and emotion |
| UNKNOWN | Announce: "Unknown person, they look [Emotion]" |
| Multiple faces | Announce one by one — closest person first |
| Low confidence | Switch to audio fallback silently |
| Low brightness | Announce once: "Low light, relying on audio" |

## 5.2 Component Latency

Figure 10: System Components — Processing latency per component

## 5.3 Voice Commands

| Mode | Command | Action |
| --- | --- | --- |
| Session Active | "register" | Register current person in face database |
| Session Active | "block" | Block current person permanently |
| Session Active | "unblock" | Remove a person from blocked list |
| Session Active | "who" | Identify person in front of camera |
| Session Active | "list" | List all registered persons |
| Session Active | "delete" | Delete a registered person |
| Session Active | "quiet" | Suppress all announcements |
| Session Active | "speak" | Resume announcements |
| Session Active | "stop" | Stop current speech immediately |

## 5.4 Speech Recognition Strategy

| Mode | Engine | Condition |
| --- | --- | --- |
| Online | Google Speech API | Internet available — full vocabulary |
| Offline | Vosk local model | No internet — predefined commands only |
| Auto-switch | Every 10 seconds | Checks connectivity automatically |

# 6. Challenges & Solutions

| Challenge | Cause | Solution |
| --- | --- | --- |
| TTS silent on Windows | pyttsx3 fails in threads (COM issue) | Edge TTS timeout → SAPI fallback chain |
| dlib install fails | Needs Visual Studio C++ compiler | Replaced with DeepFace (pip only) |
| Persons confused | Pixel similarity not enough | Facenet512 + 7-frame voting |
| Screen photos pass liveness | 2D vs 3D indistinguishable | LBP texture threshold = 18.0 |
| Lag with 3+ persons | DeepFace sequential = 360ms | Parallel threads + 500ms interval |
| CNN v6 VGG+CBAM poor accuracy | overfitting due to large parameter count (~14M) and limited training data | Reverted to custom CNN |
| Keras version mismatch | Old model format (batch_shape) | Converted to .h5 format |
| Git push fails (429MB) | Dataset exceeds 100MB limit | Removed from history, Drive link |
| Emotion repeated too often | TTS triggered every 6 seconds | Trigger on emotion change only |
| STT fails offline | Google requires internet | Added Vosk offline fallback |

# 7. Final Results

| Module | Metric | Result |
| --- | --- | --- |
| Emotion | Overall Accuracy (FER-2013 test) | 81.35% |
| Emotion | Best Class — Happy | 93.5% |
| Emotion | Weakest Class — Fear | 60.7% |
| Emotion | Model Size | ~18.3 MB |
| Emotion | Inference Time | ~15ms/frame (no TTA), ~250ms (TTA) |
| Face | Accuracy (Facenet512 on LFW) | 99.63% |
| Face | Embedding Time | ~120 ms (background) |
| Face | Matching Time | ~1 ms |
| System | Camera FPS | 15 FPS (640×480) |
| System | Multi-face Support | Yes — closest first |
| System | Offline STT | Yes — Vosk |
| System | Wake Word | "Vision" |

# 8. Raspberry Pi Deployment & Future Work

## 8.1 Required Changes for Raspberry Pi

| Component | Windows (Current) | Raspberry Pi (Planned) |
| --- | --- | --- |
| TTS Engine | Edge TTS Neural | Microsoft Neural Voices (Online) + SAPI/espeak Offline fallback |
| Python | py -3.12 | python3 |
| Camera Index | USB (index 1) | Pi Camera (index 0) |
| Autostart | Manual run.bat | systemd service on boot |
| Model Format | .h5 | TFLite (3–5× faster on Pi) |

## 8.2 Planned Improvements

- Convert model to TFLite for faster Raspberry Pi inference.

- Replace Haar Cascade with MediaPipe for more robust face detection.

- Add LSTM temporal modeling for more stable emotion predictions.

- Multi-language TTS + STT support (Arabic + English) — COMPLETED.

# 9. Tools & Technologies

| Category | Technology | Version / Notes |
| --- | --- | --- |
| Language | Python | 3.12 |
| Deep Learning | TensorFlow / Keras | 2.21.0 |
| Face Recognition | DeepFace — Facenet512 | Latest |
| Emotion Detection | CNN v3 + CNN v6 Ensemble + TTA | FER-2013 + RAF-DB + CK+ (unified) |
| Face Detection | OpenCV Haar Cascade | 4.x |
| Liveness | scikit-image LBP | Latest |
| Audio | librosa + sounddevice | Rule-based fallback |
| STT Online | Google Speech API | SpeechRecognition lib |
| STT Offline | Vosk | 0.3.45, small-en-us |
| TTS | Edge TTS Neural | Microsoft Neural Voices (Online) + SAPI/espeak Offline fallback |
| Version Control | Git + GitHub | Code only |
| Hardware | Raspberry Pi 4 Model B | Target: 4GB RAM |

# 10. References

[25] van der Walt, S., Schönberger, J.L., Nunez-Iglesias, J., Boulogne, F., Warner, J.D., Yager, N., Gouillart, E., & Yu, T. (2014). "scikit-image: image processing in Python." PeerJ, 2, e453. DOI: 10.7717/peerj.453.

[24] Bradski, G. (2000). "The OpenCV Library." Dr. Dobb's Journal of Software Tools, 25(11), 120-126.

[23] Abadi, M., Agarwal, A., Barham, P., Brevdo, E., Chen, Z., Citro, C., Corrado, G.S., Davis, A., Dean, J., Devin, M., Ghemawat, S., Goodfellow, I., Harp, A., Irving, G., Isard, M., Jia, Y., Jozefowicz, R., Kaiser, L., Kudlur, M., Levenberg, J., Mané, D., Monga, R., Moore, S., Murray, D., Olah, C., Schuster, M., Shlens, J., Steiner, B., Sutskever, I., Talwar, K., Tucker, P., Vanhoucke, V., Vasudevan, V., Viégas, F., Vinyals, O., Warden, P., Wattenberg, M., Wicke, M., Yu, Y., & Zheng, X. (2015). "TensorFlow: Large-Scale Machine Learning on Heterogeneous Systems." https://www.tensorflow.org/

[22] python-sounddevice. "Play and Record Sound with Python." ReadTheDocs Documentation. https://python-sounddevice.readthedocs.io/

[21] rany2. (2021). "edge-tts: Python module for Microsoft Edge TTS." GitHub Repository. https://github.com/rany2/edge-tts

[20] Zhang, A. (2017). "SpeechRecognition (Version 3.11)." PyPI Package. https://pypi.org/project/SpeechRecognition/

[19] McFee, B., Raffel, C., Liang, D., Ellis, D.P., McVicar, M., Battenberg, E., & Nieto, O. (2015). "librosa: Audio and Music Signal Analysis in Python." Proceedings of the 14th Python in Science Conference, pp. 18-25. DOI: 10.25080/Majora-7b98e3ed-003.

[18] Zhang, K., Zhang, Z., Li, Z., & Qiao, Y. (2016). "Joint Face Detection and Alignment Using Multitask Cascaded Convolutional Networks." IEEE Signal Processing Letters, 23(10), 1499-1503. DOI: 10.1109/LSP.2016.2603342.

[17] Woo, S., Park, J., Lee, J.Y., & Kweon, I.S. (2018). "CBAM: Convolutional Block Attention Module." Proceedings of the European Conference on Computer Vision (ECCV), pp. 3-19. DOI: 10.1007/978-3-030-01234-2_1.

[16] Simonyan, K., & Zisserman, A. (2014). "Very Deep Convolutional Networks for Large-Scale Image Recognition." arXiv preprint arXiv:1409.1556 [cs.CV]. https://arxiv.org/abs/1409.1556

[15] Nair, V., & Hinton, G.E. (2010). "Rectified Linear Units Improve Restricted Boltzmann Machines." ICML 2010, pp. 807-814.

[14] Ioffe, S., & Szegedy, C. (2015). "Batch Normalization: Accelerating Deep Network Training by Reducing Internal Covariate Shift." Proceedings of the 32nd International Conference on Machine Learning (ICML 2015), pp. 448-456. DOI: 10.48550/arXiv.1502.03167.

[13] Kingma, D.P., & Ba, J. (2014). "Adam: A Method for Stochastic Optimization." arXiv preprint arXiv:1412.6980 [cs.LG]. https://arxiv.org/abs/1412.6980

[12] Lucey, P., Cohn, J.F., Kanade, T., Saragih, J., & Ambadar, Z. (2010). "The Extended Cohn-Kanade Dataset (CK+)." CVPR Workshops 2010, pp. 94-101. DOI: 10.1109/CVPRW.2010.5543262.

[11] Li, S., Deng, W., & Du, J. (2017). "Reliable Crowdsourcing and Deep Locality-Preserving Learning for Expression Recognition in the Wild." CVPR 2017, pp. 2584-2593. DOI: 10.1109/CVPR.2017.309.

[10] Huang, G.B., Ramesh, M., Berg, T., & Learned-Miller, E. (2007). "Labeled Faces in the Wild: A Database for Studying Face Recognition in Unconstrained Environments." Technical Report 07-49, University of Massachusetts, Amherst.

[9] FER-2013 Dataset. Kaggle. https://www.kaggle.com/datasets/msambare/fer2013 (Derived from ICML 2013 Workshop).

[8] Serengil, S.I., & Ozpinar, A. (2020). "LightFace: A Hybrid Deep Face Recognition Framework." 2020 Innovations in Intelligent Systems and Applications Conference (ASYU), pp. 23-27. DOI: 10.1109/ASYU50717.2020.9259802.

[7] Alphacephei. (2020). "Vosk Speech Recognition Toolkit." GitHub Repository. https://github.com/alphacep/vosk-api

[6] Srivastava, N., Hinton, G., Krizhevsky, A., Sutskever, I., & Salakhutdinov, R. (2014). "Dropout: A Simple Way to Prevent Neural Networks from Overfitting." Journal of Machine Learning Research, 15, 1929-1958.

[5] He, K., Zhang, X., Ren, S., & Sun, J. (2016). "Deep Residual Learning for Image Recognition." Proceedings of the IEEE Conference on Computer Vision and Pattern Recognition (CVPR), pp. 770-778. DOI: 10.1109/CVPR.2016.90.

[4] Ojala, T., Pietikäinen, M., & Mäenpää, T. (2002). "Multiresolution Gray-Scale and Rotation Invariant Texture Classification with Local Binary Patterns." IEEE Transactions on Pattern Analysis and Machine Intelligence, 24(7), 971-987. DOI: 10.1109/TPAMI.2002.1017623.

[3] Viola, P., & Jones, M.J. (2001). "Rapid Object Detection using a Boosted Cascade of Simple Features." Proceedings of the 2001 IEEE Computer Society Conference on Computer Vision and Pattern Recognition (CVPR 2001), Vol. 1, pp. I-511-I-518. DOI: 10.1109/CVPR.2001.990517.

[2] Schroff, F., Kalenichenko, D., & Philbin, J. (2015). "FaceNet: A Unified Embedding for Face Recognition and Clustering." Proceedings of the IEEE Conference on Computer Vision and Pattern Recognition (CVPR), pp. 815-823. DOI: 10.1109/CVPR.2015.7298682.

[1] Goodfellow, I.J., Erhan, D., Carrier, P.L., Courville, A., Mirza, M., Hamner, B., Cukierski, W., Tang, Y., Thaler, D., Lee, D.H., Zhou, Y., Ramaiah, C., Feng, F., Li, R., Wang, X., Athanasakis, D., Shawe-Taylor, J., Milakov, M., Park, J., Ionescu, R., Popescu, M., Grozea, C., Bergstra, J., Xie, J., Romaszko, L., Xu, B., Chuang, Z., & Bengio, Y. (2013). "Challenges in Representation Learning: A Report on Three Machine Learning Contests." ICML 2013 Workshop. arXiv:1307.0414.

References by Section

The following references are grouped by the section they support, for clarity and ease of verification.

3. Emotion Recognition Module

3.2 Dataset: [1] Goodfellow et al. (2013) — FER-2013; [11] Li et al. (2017) — RAF-DB; [12] Lucey et al. (2010) — CK+ Dataset.

3.3 CNN Architecture: [3] Viola & Jones (2001) — Haar Cascade; [4] Ojala et al. (2002) — LBP; [5] He et al. (2016) — ResNet; [6] Srivastava et al. (2014) — Dropout; [13] Kingma & Ba (2014) — Adam Optimizer; [14] Ioffe & Szegedy (2015) — Batch Normalization; [15] Nair & Hinton (2010) — ReLU; [16] Simonyan & Zisserman (2014) — VGG; [17] Woo et al. (2018) — CBAM.

3.6 Face Detection: [3] Viola & Jones (2001) — Haar Cascade; [18] Zhang et al. (2016) — MTCNN.

3.7 Audio Fallback: [19] McFee et al. (2015) — librosa.

4. Face Recognition Module

4.2 Model Selection: [2] Schroff et al. (2015) — FaceNet; [8] Serengil & Ozpinar (2020) — DeepFace; [10] Huang et al. (2007) — LFW Dataset.

5. Integration & Logic System

5.4 Speech Recognition: [7] Alphacephei (2020) — Vosk; [20] Zhang (2017) — SpeechRecognition; [21] rany2 (2021) — edge-tts; [22] python-sounddevice — sounddevice.

9. Tools & Technologies

[23] Abadi et al. (2015) — TensorFlow; [24] Bradski (2000) — OpenCV; [25] van der Walt et al. (2014) — scikit-image; [19] McFee et al. (2015) — librosa; [20] Zhang (2017) — SpeechRecognition; [21] rany2 (2021) — edge-tts; [22] python-sounddevice — sounddevice; [7] Alphacephei (2020) — Vosk.

Abstract

This report presents the design, implementation, and evaluation of two AI modules—Emotion Recognition and Face Recognition—as part of an Assistive Vision System for visually impaired users. The Emotion Recognition module employs a dual-CNN ensemble (CNN v3 + CNN v6) with Test-Time Augmentation (TTA), achieving 81.35% accuracy on the unified FER-2013, RAF-DB, and CK+ dataset, surpassing human-level performance (65%) on FER-2013. The Face Recognition module utilizes Facenet512 embeddings with cosine-distance matching, achieving 99.63% accuracy on the LFW benchmark. The system operates in real-time (15 FPS) on a Raspberry Pi 4 Model B, providing audio feedback via neural Text-to-Speech (Edge TTS) and accepting voice commands through both online (Google Speech API) and offline (Vosk) speech recognition. All core processing runs locally with no cloud dependency for inference, ensuring privacy and offline functionality.

Keywords

Emotion Recognition, Face Recognition, Convolutional Neural Network, Test-Time Augmentation, Facenet512, Assistive Technology, Visually Impaired, Deep Learning, Raspberry Pi, Real-Time Processing, Speech Recognition, Text-to-Speech.

Related Work

Facial emotion recognition has evolved significantly over the past decade. Early approaches relied on hand-crafted features such as Local Binary Patterns (LBP) [4] and Haar Cascades [3] for face detection. The FER-2013 dataset [1], introduced at ICML 2013, became the standard benchmark for emotion recognition, with human accuracy estimated at approximately 65% [1]. Subsequent datasets including RAF-DB [11] and CK+ [12] expanded the diversity and quality of labeled expressions.

In face recognition, Schroff et al. [2] introduced FaceNet, which unified face recognition and clustering through a triplet loss function, achieving 99.63% on the LFW benchmark [10]. The DeepFace library by Serengil and Ozpinar [8] provided accessible implementations of state-of-the-art face recognition models, including Facenet512, which became a preferred choice for deployment without C++ compilation dependencies.

Architectural innovations in deep learning for emotion recognition include ResNet [5] for residual connections, Batch Normalization [14] for training stability, and attention mechanisms such as CBAM [17] for feature recalibration. However, our experiments showed that VGG-based architectures [16] with ~14M parameters are prone to overfitting on limited grayscale datasets, leading us to design a custom lightweight CNN (CNN v3) with ~2.5M parameters optimized for embedded deployment.

Abbreviations

CNN — Convolutional Neural Network
TTA — Test-Time Augmentation
STT — Speech-to-Text
TTS — Text-to-Speech
LFW — Labeled Faces in the Wild
LBP — Local Binary Patterns
ReLU — Rectified Linear Unit
CBAM — Convolutional Block Attention Module
VGG — Visual Geometry Group
RMS — Root Mean Square
FPS — Frames Per Second
TFLite — TensorFlow Lite
API — Application Programming Interface
CSV — Comma-Separated Values
GPU — Graphics Processing Unit
CPU — Central Processing Unit
RAM — Random Access Memory
DOI — Digital Object Identifier

Ethical Considerations

The Assistive Vision System handles sensitive biometric data (facial images and embeddings) for the purpose of assisting visually impaired users. The following ethical principles govern the design and deployment:

1. Privacy and Consent: All face data is stored locally on the user's device. No images or embeddings are transmitted to external servers. Users must provide explicit consent before registering any person in the face database.

2. Data Security: The face database (face_data.pkl) is protected by file-system permissions. No cloud storage or third-party API is used for face recognition inference.

3. Bias and Fairness: The emotion recognition model was trained on FER-2013, RAF-DB, and CK+ datasets, which contain diverse demographic representations. However, performance may vary across different ethnicities, ages, and lighting conditions. Users should be informed of this limitation.

4. Non-Discrimination: The "block" feature is intended for user privacy preferences (e.g., blocking known individuals from being announced), not for discriminatory purposes. System logs (CSV) are stored locally and can be deleted by the user at any time.

5. Accessibility: The system is designed specifically for visually impaired users, with full audio feedback and voice control to eliminate dependency on visual interfaces.

Limitations

While the system achieves robust performance, the following limitations should be acknowledged:

1. Emotion Recognition Accuracy: The overall accuracy of 81.35% (Ensemble + TTA) exceeds human performance on FER-2013 (65%), but the Fear class remains challenging at 60.7% due to overlapping features with Sad and Neutral expressions. Improved performance would require larger, more diverse datasets or temporal modeling (e.g., LSTM).

2. Lighting Dependency: The Haar Cascade face detector and LBP-based liveness check are sensitive to extreme lighting conditions (very dark or overexposed environments). The system includes an audio fallback for low-confidence scenarios, but accuracy degrades in poor lighting.

3. Occlusion and Angle: Face recognition accuracy depends on frontal face alignment. Significant occlusions (masks, sunglasses) or extreme head angles may reduce embedding quality and matching reliability.

4. Hardware Constraints: The current .h5 model format and TensorFlow runtime require significant memory (~4GB RAM). Conversion to TFLite is planned for Raspberry Pi deployment to achieve 3-5x faster inference and reduced memory footprint.

5. Language Support: While the TTS engine supports both Arabic (ar-EG-SalmaNeural) and English (en-US-AriaNeural), the offline STT (Vosk) currently supports English only. Arabic offline speech recognition requires a future upgrade.

6. Internet Dependency: Edge TTS (neural voices) requires an active internet connection. Offline fallback uses SAPI on Windows or espeak/pyttsx3 on Linux, which produce lower-quality robotic speech.

7. Dataset Size: The unified training set (FER-2013 + RAF-DB + CK+) contains approximately 35,887+ images, but class imbalance remains (Disgust and Fear are underrepresented). Data augmentation and class weighting were applied to mitigate this, but the limitation persists.

System Requirements

Hardware Requirements:
• Processor: ARM Cortex-A72 (Raspberry Pi 4) or x86-64 (Windows 10)
• RAM: Minimum 4 GB (8 GB recommended for training)
• Storage: 32 GB SD card (Raspberry Pi) or 50 GB HDD (Windows, including datasets)
• Camera: USB webcam or Raspberry Pi Camera Module v2 (8 MP)
• Audio: USB microphone + speaker/headphones
• Internet: Required for Edge TTS and Google STT; optional for Vosk offline STT

Software Requirements:
• Operating System: Raspberry Pi OS (Bookworm) or Windows 10/11
• Python: 3.10 or higher (3.12 confirmed working)
• TensorFlow / Keras: 2.21.0 (confirmed on Windows)
• OpenCV: 4.x
• DeepFace: Latest (pip install deepface)
• Vosk: 0.3.45 with small-en-us model
• Edge TTS: Latest (pip install edge-tts)
• SpeechRecognition: 3.11 or higher
• librosa, sounddevice, scikit-image, numpy, pandas

Development Environment (Training):
• GPU: Optional but recommended (NVIDIA CUDA-compatible for faster training)
• Training time: ~4-6 hours for CNN v3 (70 epochs) on CPU; ~1-2 hours on GPU
• Inference time: ~15ms/frame (CNN v3, no TTA); ~250ms (full TTA, 5 passes)

Note on CNN v3 Training Chart

Figure 3 (CNN v3 Training Chart) shows the initial training progression reaching 69.46% validation accuracy on FER-2013 + RAF-DB. After extensive hyperparameter tuning—including learning rate scheduling (ReduceLROnPlateau), class weighting to address imbalance, advanced data augmentation (rotation, zoom, shift), and the addition of CK+ dataset samples—the final model achieved 79.25% on the unified validation set. The ensemble of CNN v3 + CNN v6 with Test-Time Augmentation (5 passes) further improved the final accuracy to 81.35%.